October 20, 2009

STANDARDS FOR PLUMBING FIXTURES AND ACCESSORIES

|  | PRE-K | K-5 ADA | K-5 (Non ADA) Group Toilet Rooms | ADA Adults | Adults |
| --- | --- | --- | --- | --- | --- |
| LAVATORY | 22”AFF | 31”AFF (top of rim) 24” Apron/Knee Clearance | 31”AFF (top of rim) | 34”AFF (top of rim) 27” Apron/Knee Clearance | 34”AFF (top of rim) |
| WATER CLOSET | *Standard | *Standard | *Standard | 17-19” top of Seat | Standard |
| GRAB BAR** | 20” | 25” to cl. | Not Required | 35” to cl. | Not Required |
| WATER COOLER | 22” max. top of spout | 30”max. top of spout | 36” top of spout | 36”max. top of spout | 42” top of spout |
| URINALS | NA | 14” top of rim | 17” top of rim | 17” top of rim | 24” top of rim |
| MIRRORS | 26”AFF (above Lavatory) | 36”AFF (above lavatory) | 34”AFF to bottom of mirror | 40”max. AFF (above Lavatory) | 40”AFF to bottom of mirror |

Coordinate all information with Section 15410 – Plumbing Fixtures (latest revision) of the SBBC Material Standards.

All Lavatories are to be mounted a minimum of  15” to the centerline of the lavatory  from the finished sidewall.

TOILET TISSUE DISPENSER– For Elementary schools  ADA usage –unit to be mounted 7”-9” from the front edge   of the water closet to the centerline  of the unit and 12” above the  grab bar elevation to the bottom of unit.   For all non ADA toilet stalls, dispensers are to be mounted 36” from rear wall to centerline of unit in all cases and 30”AFF to bottom of unit for elementary school children’s usage,  and,   36”AFF to bottom of unit for adult usage.  For adult ADA usage – unit is to be  a  G.P. Jumbo Jr.  type Bathroom Tissue Dispenser  (maximum 11’’ dispenser  diameter)  mounted  36” from the rear wall to the centerline of the unit and 3” clear from the bottom of the grab bar to the top of the dispenser ( see illustration).

ELEMENTARY SCHOOL CLASSROOM COUNTER TOPS (with sink and bubbler): Pre-K through 5 -  counter top at 26”AFF  with 30” max. to bubbler spout. Bubblers shall be installed providing  side wheelchair access to the bubbler and the sink faucets.  Indicate clear wheelchair access to both the bubbler and the faucets.

### ELEMENTARY  SCHOOL CLASSROOM COUNTER TOPS (without bubblers):  Pre-k through grade 5 – 28”AFF.  (All specialty classrooms to be fabricated per the Ed Specs.)

##### MIRRORS in individual toilet rooms and in all ADA stalls:  18” x 30” mounted above the Lavatory .

### MIRRORS  (18” x 30”) in group toilet rooms are to be hung on wall opposite the sink area and  mounted  as noted above.

ACCESSORIES  in Elementary Schools are to be mounted with their dispensing mechanisms, start buttons, coin slots or dispenser openings located 36”AFF.  Accessories for adult use are to be mounted per ADA specifications.

*       Water closet centerline: Grades K-5 to be 16” from side wall.  All adult water closets are to be  centered at 18”max.  from side wall.

Water closets in elementary schools in student applications, are to be 10” rough fixtures roughed in at 12”.

**      Grab Bars: 1 ¼” O.D. Diam. Max..  Side wall grab bar – 42” long mounted 12” from rear wall; rear wall grab bars to be 36” long.   A 16” Flush Valve is to be used with all K-5 ADA water closets.  This, along with the additional 2” roughing will allow  the installation of a continuous 36”rear grab bar @ 25”AFF elevation at the rear wall of the water Closet.

An 11 ½” flush valve shall  be used at all  water closets which are either for adults (all applications)  or  children’s non  ADA accessible..

Elementary School Media Center Toilet Rooms are to follow the K-5 ADA standards.

The ADA stalls in the “Boys” and “Girls”  toilet rooms are to follow  the elementary school K-5ADA standards.